- What is SOHO network?

A Small Office/Home Office (SOHO) network is a local area network (LAN) that’s dewsigned for small business and people working from home or small offices and Can be used for remote work also.

- What is NAT?

Network Address Translation (NAT) is a service that operates on a router or edge platform to connect private networks to public networks like the internet.

That allows private IP networks to use the internet and cloud.

- What is PAT?

Port Address Translation (PAT) is a type of network address translation (NAT) that maps a network’s private internal IPV4 addresses to a single public IP address.

- Different between NAT & PAT?

The primary distinction is that NAT is used to map public IP addresses to private IP addresses in a one to one or many to one  relationship.

On the other hand , PAT is a sort of NAT in which numerous private IP addresses (many to one) are mapped into a single public IP address via ports.

- What Is Acl?

An access control list (ACL) is a security mechanism that defines who has access to bucket , objects , networks , storage , and files , and what level of access they have.

- What Are Different Types of Acl? What Is Wildcard Mask?

There are two types of ACL :-

- Filesystem ACLs

- Networking ACLs

Other types :-

- Standard ACLs

- Extended ACLs

- Dynamic ACLs

- Reflexive ACLs

Wildcard Mask :-

A wildcard mask is a bit mask that indicates  which parts of an IP address re available for examination. Its used to match specific IP addresses or range of IP addresses and can be used in several ways like ,

Cisco IOS, Cisco access control list, Large networks, Firewalls.

- Explain Circuit switching.

Circuit switching is a network configuration that establishes a dedicated physical path between two end points for the duration of a connections.

This method of communication is often used for long distance calls or communication sessions.

- What is difference between leased line and broadband?

A dedicated leased line offers higher reliability with its consistent data connectivity.

Broadband offers varying connections based on certain factors that may be prone to downtime.

- Difference between a POTS line and a leased line?

A Plain Old Telephone Service (POTS) line is an analog phone system that uses copper wires to transmit voice over a network, while a leased line is a private telecommunication circuit that can provide a dedicated, high speed internet connection.

- Practice on printer sharing.

Done in lab.

- Use of IIS.

Internet information services (IIS) is a Microsoft web server that runs on windows and is used to exchange web content with internet users.

Can be used to host, deploy, and manage web application on windows operating systems.

- Create FTP server.

Done in lab.

- What is the difference between cloud and virtualization?

Virtualization is a technology that allows you to create multiple simulated environments or  dedicated resources from a single, physical hardware system, which cloud are IT environments that abstract, pool, and share scalable resources across a network.

- Why are network monitoring tools used?

Network monitoring tools help you monitor your network infrastructure to identify and fix issues.

They collect data from network devices using network protocols to track performance metrics like traffic, bandwidth, and availability.

They can also help keep your network safe from threats.

- What is ping ?

Ping is a computer network tool that measures the time it takes for a small amount of data to travel from a device to a server on the internet and back again.

Its also known as latency which is a more technical term. Ping time is measured in milliseconds(ms).

- What is traceroute ?

Traceroute is a network command that helps diagnose problems in a network path by tracing the path that IP packets take from one computer to another.

- What is nslookup?

The nslookup tool is a network administration command line tool used for querying DNS records to obtain domain names or IP address mapping or for any other specific records. Its valuable resources for server testing and troubleshooting DNs issues.

- Explain core switches.

A core switch is a high performance network switch that sits at the top of a networks structure and serves as the central point of connectivity.

Are responsible for data transfer, reliability, security.

- What is network management?

Network management is a process that involves configuring, monitoring, and managing a network’s performance.

Network management is the sum total of application, tools, and processes used to provision, operate, maintain, administer and secure network infrastructure.

- Explain Event Viewer.

The event viewer is a tool in Microsoft windows that provides detailed information about events that occur on a computer.

Event viewer acts as a centralized log management system, collecting logging information from various sources within the operating system.

- Practice "parental control" or "family safety" option in control panel. What are network vulnerabilities?

Done in lab.

Network vulnerabilities are flaws in a networks hardware, software, or processes that can lead to security breaches and threaten the networks normal functioning.  They can be physical or non physical.

- What are the types of network security attacks?

There are some types of attacks like,

Dos and DDoS attack

Phishing attack

Ransomware attack

XSS attack

Trojan horses attack…..